Rule-of-Thumb formulas for flight calculations

Calculating bank angles for standard rate turns

15% of TAS (in knots)

or, identically,

10% of TAS +  ½ of that number

Example:

TAS = 120 knots

10% of 120 = 12

½ of 12 = 6

==> 12 + 6 = 18 degrees angle of bank

Calculating roll-out from steep turns (>= 45 degree bank angle)

Initiate roll-out at a heading equivalent to 1/2 the bank angle.

Examples:

45 degree bank

==> 45° / 2  = 22.5°

i.e., making a steep turn turn to the left to a heading of  360°, begin rollout as turn reaches 023°

Making a steep turn turn to the right to a heading of  360°, begin rollout as turn reaches 337°

Calculating roll-out from normal bank-angle (e.g., standard rate) turns

Initiate roll-out at a heading equivalent to 1/3  the bank angle.

Example:

18 degree bank (standard rate at 120 knots)

==> 18° / 3 = 6°

i.e., making an 18°  banked turn to the right to a heading of 360°, begin rollout as turn reaches 354°

Calculating level-off from climb or descent

10% of Rate of Climb/Descent

Example:

Rate of climb: 500 fpm

==>  500 fpm / 10 = 50 feet lead to assigned altitude.

Calculating pivotal altitude for pylon-eights

square TAS (in knots) / 11.3

Example:

True airspeed (e.g., Va): 90 knots

==> (90 * 90) = 8100 / 11.3 = 716.8 feet

i.e., enter the pylon-eight at 720 feet in a descending turn upwind

N.B., using TAS-squared/10 does not gives a close-enough approximation:

==> (90 * 90) / 10 = 810 feet, 100 feet too high.

Calculating lead intercepts (in nautical miles) for DME arcs or any selected course

TAS (knots) / 200

Example:

TAS = 120 knots

==> 120 / 200 = 0.6 nm lead

Initiate a standard rate turn 0.6nm prior to arriving at the arc (at a 90 degree intercept to the arc).

For a 45 degree intercept [i.e., ½ of 90 degrees], (e.g., to an airway), use one-half the lead for 90 degrees (initiate at 0.3nm).

For a 30 degree intercept (e.g., on a radar vector to an ILS final approach course), use one-third the lead for 90 degrees (initiate at 0.2nm).

Calculating Va: (maneuvering speed) at weights less than maximum gross weight (MGW)

Square root of (Actual [current] weight  / MGW) * published Va (162mph)

Example:

MGW: 3,600

Published Va: 162 knots

Actual [current] weight: 3,400

==> Square root of (3,400/3,6000) = 0.972  *  162 = 157.44 knots

Va at 3,400 = 157 knots